Opis predmetu zákazky

Názov zákazky: Automatizovaná technologická jednotka pre destiláciu silíc

1. Všeobecná charakteristika

Predmetom zákazky je dodávka, inštalácia a zaškolenie obsluhy pre plne automatizovanú, multifunkčnú destilačnú jednotku určenú na parnú destiláciu rastlinných silíc (esenciálnych olejov a hydrolátov). Zariadenie musí byť konštruované ako mobilné a napájané výhradne elektrickou energiou.

2. Technická špecifikácia a parametre

A. Kotlová jednotka a ohrev

- Pracovný objem kotla: minimálne 100 litrov.

- Materiál: Vysokokvalitná nerezová oceľ (min. AISI 304/316) s tepelnou izoláciou pre elimináciu tepelných strát a zvýšenie bezpečnosti.

- Zdroj tepla: Integrovaný elektrický ohrev s plynulou reguláciou výkonu.

- Miešací systém: Integrované mechanické miešadlo s motorovým pohonom.

B. Destilačná nadstavba a extrakcia

- Modul pre parnú destiláciu (Extrakcia): Externý alebo integrovaný modul (extractor) s objemom prispôsobeným kapacite kotla, určený na parnú destiláciu biomasy (napr. levanduľa) bez priameho kontaktu s vriacou vodou.

- Katalýza: Zariadenie musí obsahovať vyberateľné medené prvky (katalytické vložky) v ceste pár na odstránenie nežiaducich zlúčenín síry.

- Chladiaci systém: Vysokoúčinný kondenzátor s automatickou reguláciou prietoku chladiacej vody.

C. Automatizácia a riadenie

- Riadiaci systém: Plne automatizovaný systém s PLC riadením a dotykovým rozhraním.

- Funkcie riadenia:

  - Automatické riadenie teploty, tlaku a energetického príkonu.

  - Možnosť ukladania a spúšťania programovateľných receptov pre rôzne druhy surovín.

  - Automatické oddeľovanie frakcií (rezy) pri destilácii bez nutnosti manuálneho zásahu.

  - Vzdialený prístup a monitorovanie procesu (napr. cez Wi-Fi/Cloud rozhranie).

D. Mobilita a príslušenstvo

- Mobilita: Celá zostava musí byť umiestnená na mobilnom podvozku s brzdenými kolieskami pre ľahkú manipuláciu v rámci prevádzky.

- Separácia: Dodávka musí obsahovať zariadenie na gravitačné oddeľovanie esenciálneho oleja od hydrolátu (Florentská váza / laboratórny separátor).

3. Požiadavky na dodávku

- Komplexnosť: Dodávka „na kľúč“ (turn-key), vrátane montáže a oživenia u objednávateľa.

- Dokumentácia: Technický list, návod na obsluhu a prehlásenie o zhode (CE certifikácia).

- Záruka: Minimálne 24 mesiacov.

Description of the Subject of the Contract

Contract Title: Automated Technological Unit for Essential Oil Distillation

1. General Characteristics

The subject of the contract is the delivery, installation, and operator training for a fully automated, multifunctional distillation unit designed for the steam distillation of plant-based essences (essential oils and hydrosols). The equipment must be designed as a mobile unit and powered exclusively by electricity.

2. Technical Specifications and Parameters

A. Boiler Unit and Heating

- Boiler Working Capacity: Minimum 100 liters.

- Material: High-quality stainless steel (min. AISI 304/316) with thermal insulation to eliminate heat loss and increase safety.

- Heat Source: Integrated electric heating with continuous power regulation.

- Mixing System: Integrated mechanical agitator with a motor drive.

B. Distillation Superstructure and Extraction

- Steam Distillation Module (Extraction): External or integrated module (extractor) with a volume adapted to the boiler capacity, designed for steam distillation of biomass (e.g., lavender) without direct contact with boiling water.

- Catalysis: The device must include removable copper elements (catalytic inserts) in the vapor path to remove undesirable sulfur compounds.

- Cooling System: High-efficiency condenser with automatic regulation of cooling water flow.

C. Automation and Control

- Control System: Fully automated system with PLC control and a touchscreen interface.

- Control Functions:

  - Automatic control of temperature, pressure, and energy input.

  - Capability to store and run programmable recipes for various types of raw materials.

  - Automatic separation of fractions (cuts) during distillation without the need for manual intervention.

  - Remote access and process monitoring (e.g., via Wi-Fi/Cloud interface).

D. Mobility and Accessories

- Mobility: The entire assembly must be mounted on a mobile chassis with braked casters for easy manipulation within the facility.

- Separation: The delivery must include a device for gravitational separation of essential oil from hydrosol (Florentine flask / laboratory separator).

3. Delivery Requirements

- Complexity: "Turn-key" delivery, including assembly and commissioning at the customer's site.

- Documentation: Technical data sheet, operating manual, and declaration of conformity (CE certification).

- Warranty: Minimum 24 months.

NÁVRH NA PLNENIE KRITÉRIA/ PROPOSAL FOR CRITERIA FULFILLMENT

| Obchodné meno uchádzača/ Business Name of the Bidder: |  |
| --- | --- |
| Adresa sídla uchádzača/ Registered Address of the Bidder: |  |
| Meno oprávnenej osoby podpisovať za firmu/ Name of the Person Authorized to Sign: |  |
| Meno kontaktnej osoby a jej funkcia/ Contact Person and Title: |  |
| Číslo telefónu kontaktnej osoby/ Contact Person’s Phone Number: |  |
| E-mail kontaktnej osoby/ Contact Person’s E-mail: |  |

| Kritérium/ Criterion | Cena bez DPH/ Price excluding VAT | DPH/VAT | Cena s DPH/ Price including VAT |
| --- | --- | --- | --- |
| Celková zmluvná cena za predmet zákazky spolu (EUR)/ Total Contract Price for the Subject of the Contract (EUR) | ................. EUR | ................. EUR | ................. EUR |

V/In......................... dňa/Date ...

................................................