Technical specification of the subject of the contract

Contract name: Anti-frost wind turbines with automatic control system Kemtax Ltd.​

- Anti-freeze wind turbines ... 1 pc

The machine serves to protect trees from frost, the basic function is that the driven blade generates a flow of air (slightly warmer) from higher altitudes downwards and thus warmer air is achieved at the treetops than without the use of wind turbines. In this way, an increase in ground temperature by 4-5 C can be achieved. The machine consists of a tower, a rotor, an engine and a drive system between the engine and the rotor.

| Manufacturer, type/brand of the offered goods: |  |  |
| --- | --- | --- |
| Parameter description | Minimum parameter requirements | Your parameter (state meets / does not meet or state value) |
| Tower material | Galvanized steel |  |
| Tower diameter/wall thickness | Min. 500 mm / min. 0.8 mm |  |
| Tower height | Min. 10 m - max. 12 m |  |
| Shovel - size | Min. 5.5 m - max. 7 m |  |
| Bucket material | Fiberglass or carbon fiber |  |
| Engine – aggregate | Diesel generator |  |
| HP performance | Min. 140 HP – max. 180 HP |  |
| Fuel tank | At least 400 liters |  |
| Protective mesh against pests and birds | Required |  |
| Drive system: | Gearbox connected to cardan shaft |  |
| Overload protection | Required |  |
| Rotor rotation around the axis 360 | Rotation around the axis in min. 4 – max. 7 minutes |  |
| Territory protection (in ha) | Min. 6 hectares |  |
| Accessories: auxiliary heating via electric fan heater, quantity: 1 unit | Electric heater, min. 9 kW max. 15 kW |  |

*Compliance with the requirement - the applicant shall indicate by selecting from the following options:

- the offer meets the specified technical parameter - the bidder states that it meets

- the offer does not meet the specified technical parameter - the bidder states it does not meet

- for each numerical parameter, the applicant shall indicate the exact parameter

Note, warning: the offer is accepted only if all parameters are met!

The price does not include the implementation of the foundation plate. The foundation plate is implemented by the customer based on the technical sheet of the wind turbine supplier.

Installation of equipment on site as determined by the supplier in accordance with the project documentation on the affected plots number: KN C 733/1, 704, 853/2, 686/1, cadastral area: Buzitka.

I also declare that the price offer includes delivery/transportation and installation of the subject of the contract and training of the operator at the contracting authority's location.

- Automatic control system - for anti-freeze wind turbines ... 1 pc

| Manufacturer, type/brand of the offered goods: |  |  |
| --- | --- | --- |
| Parameter description | Minimum parameter requirements | Your parameter (state meets / does not meet or state value) |
| automatic (automatic on and off according to the set temperature) | Required |  |
| Photovoltaic panel and accumulator | Min. 15 W panel and / min. 105 Ah accumulator |  |
| Weather monitoring | Min. heat and wind sensor |  |
| intelligent online connection and management | Required |  |

*Compliance with the requirement - the applicant shall indicate by selecting from the following options:

- the offer meets the specified technical parameter - the bidder states that it meets

- the offer does not meet the specified technical parameter - the bidder states it does not meet

- for each numerical parameter, the applicant shall indicate the exact parameter

Note, warning: the offer is accepted only if all parameters are met!

Installation of equipment on site as determined by the supplier in accordance with the project documentation on the affected plots number: KN C 733/1, 704, 853/2, 686/1, cadastral area: Buzitka.

I also declare that the price offer includes delivery/transportation and installation of the subject of the contract and training of the operator at the contracting authority's location.

| Name and surname statutory representative: |  |
| --- | --- |
| Signature and stamp: |  |
| Place and date of signature: |  |