Technical specification of the subject of the contract - assignment

Contract name: Walnut shaking and harvesting equipment

- Tree shaking device ... 1 pc.

| Manufacturer, type/brand of the offered goods: |  |  |
| --- | --- | --- |
| Parameter description | Minimum parameter requirements | Your parameter (state meets / does not meet or state value) |
| Mobile version | Connection to the cardan shaft tractor |  |
| Performance (number of trees/hour) | Min. 100 trees / hour |  |
| Tractor power requirements: | Min 70 Horsepower max. 120 Horsepower |  |
| Vibrating head | Hydraulic adjustable, telescopic design |  |
| Tree trunk diameter (min. jaw opening for gripping the tree) | at least 500 mm |  |
| Head shift (from wheel to tree) | Min. 2 m |  |
| Height of shaking | Min. From 0.5 m to min. 3 m |  |
| Telescopic arm shift | Min. 1 m |  |
| Control | Electronically, lever or joystick from the tractor cabin |  |

*Compliance with the requirement - the applicant shall indicate by selecting from the following options:

- the offer meets the specified technical parameter - the bidder states that it meets

- the offer does not meet the specified technical parameter - the bidder states it does not meet

- for each numerical parameter, the applicant shall indicate the exact parameter

Note, warning: the offer is accepted only if all parameters are met!

I also declare that the price offer includes delivery/transport of the subject of the contract and training of the service personnel at the contracting authority's location.

- Automatic walnut harvesting equipment ... 2pcs

| Manufacturer, type/brand of the offered goods: |  |  |
| --- | --- | --- |
| Parameter description | Minimum parameter requirements | Your parameter (state meets / does not meet or state value) |
| Basic characteristics | Automatic, self-propelled harvesting equipment nuts |  |
| Engine power / fuel | Min. 75 HP / diesel |  |
| Drive | 4x4 |  |
| Shot width | Min. 2 m |  |
| Performance, collection capacity | Min. 1 ha/hour. |  |
| Sorting: | Min. dirt separation |  |
| Tank volume | Min. 1.2 m3 - max. 5 m3 |  |
| Tray control | Hydraulically |  |
| Cabin with roof and air conditioning | Required |  |
| Harvesting other nuts | Min. walnut, almond, hazelnut |  |
| Automation, digitalization, robotization: |  |  |
| Control | Electronic control from the cab |  |
| Monitoring | At least 2 cameras |  |
| Navigation | Automatic guidance with accuracy up to +-3 cm. , GNSS +INS navigation systems, display, electric steering wheel |  |
| Monitoring, logging, alerts | Information about the engine, consumption, performance, distance traveled, error messages |  |

*Compliance with the requirement - the applicant shall indicate by selecting from the following options:

- the offer meets the specified technical parameter - the bidder states that it meets

- the offer does not meet the specified technical parameter - the bidder states it does not meet

- for each numerical parameter, the applicant shall indicate the exact parameter

Note, warning: the offer is accepted only if all parameters are met!

I also declare that the price offer includes delivery/transport of the subject of the contract and training of the service personnel at the contracting authority's location.

| Name and surname statutory representative: |  |
| --- | --- |
| Signature and stamp: |  |
| Place and date of signature: |  |